GeoDev – Week 2 Assignment

Data Note: Project Datasets Downloaded and Opened in QGIS

Seigha Tukoroa  |  18 October 2026

GitHub repository: [https://github.com/seighatukoroa/geodev-lab-project.git]

Note: Yellow-highlighted items are values only you can read from QGIS (feature counts, scene IDs, dates). Fill them in, then remove the highlight before submitting.

# 1. Project Context

This project uses satellite imagery and vector reference layers to study the Niger Delta, Nigeria, with a focus on mapping infrastructure such as pipeline corridors. Optical imagery (Landsat 8/9 and Sentinel-2) supports land cover and corridor identification. Radar imagery (Sentinel-1) adds all-weather coverage, which matters in a cloudy delta region. Vector layers (administrative boundaries and airports) provide the reference geography for clipping, labelling and context.

Study area: [e.g. Niger Delta / Bayelsa–Delta–Rivers corridor]   Tools: QGIS [version], Git/GitHub.

# 2. Dataset Summary

| Dataset | Source | Type | Role in project |
| --- | --- | --- | --- |
| Landsat 8/9 | USGS EarthExplorer | Raster | Optical imagery, 30 m |
| Sentinel-2 | Copernicus Data Space | Raster | Optical imagery, 10–60 m |
| Sentinel-1 | Copernicus Data Space | Raster (SAR) | Radar imagery, all-weather |
| Nigeria airports | HDX | Vector (points) | Reference infrastructure |
| Nigeria admin boundaries | GRID3 | Vector (polygons) | Primary admin boundaries |
| Nigeria admin boundaries | DIVA-GIS | Vector (polygons) | Alternative/cross-check boundaries |

# 3. Dataset Descriptions

## 3.1 Landsat 8/9 (USGS)

| Source | USGS EarthExplorer (see also Landsat mission page) |
| --- | --- |
| Product | Landsat 8/9 OLI/TIRS, Collection 2 [Level-1 or Level-2 – confirm] |
| Number of features | Raster data, so there are no features. Scenes downloaded: [N]. Scene IDs/path-row: [e.g. 188/056]. Acquisition dates: [dates] |
| Geometry type | Raster grid (no vector geometry) |
| Key columns | No attribute table. The key fields are the bands: B1 (coastal), B2–B4 (blue, green, red), B5 (NIR), B6–B7 (SWIR), B10 (thermal), plus the _MTL.txt metadata file (date, sun angle, cloud cover). |
| CRS / resolution | UTM, WGS84 [zone – check in QGIS]; 30 m multispectral. |
| Gaps and issues | Cloud and cloud shadow, which are common over the Niger Delta. Scene edges contain fill values (0/NoData). Revisit is 16 days per satellite. Scene cloud cover: [%]. |

## 3.2 Sentinel-2 (Copernicus)

| Source | Copernicus Browser (Copernicus Data Space Ecosystem); mission information: ESA Sentinel-2 |
| --- | --- |
| Product | Sentinel-2 MSI [Level-2A (surface reflectance) or Level-1C – confirm] |
| Number of features | Raster data, so there are no features. Tiles/scenes downloaded: [N]. Tile ID: [e.g. T32NNK]. Dates: [dates] |
| Geometry type | Raster grid (no vector geometry) |
| Key columns | No attribute table. The key fields are the bands: B2, B3, B4, B8 at 10 m; B5, B6, B7, B8A, B11, B12 at 20 m; B1, B9, B10 at 60 m. L2A also includes the Scene Classification Layer (SCL). |
| CRS / resolution | UTM, WGS84 [zone]; 10 m, 20 m and 60 m depending on band. |
| Gaps and issues | Cloud and cloud shadow, haze over the delta, and tile edges with no data. Bands come at different resolutions and need resampling for band stacks. Revisit is about 5 days. |

## 3.3 Sentinel-1 (Copernicus)

| Source | Copernicus Browser; mission information: ESA Sentinel-1 |
| --- | --- |
| Product | Sentinel-1 C-band SAR, [GRD, IW mode – confirm] |
| Number of features | Raster data, so there are no features. Scenes downloaded: [N]. Dates: [dates]. Orbit direction: [ascending/descending] |
| Geometry type | Raster grid (no vector geometry) |
| Key columns | No attribute table. The key fields are the polarisation bands (VV and VH), which hold backscatter intensity, plus the acquisition metadata. |
| CRS / resolution | Ground range, WGS84 [confirm]; about 10 m pixel spacing in IW GRD. |
| Gaps and issues | Speckle noise, radar shadow and layover (limited in flat terrain), and no spectral colour information. Raw GRD needs pre-processing (orbit file, calibration, terrain correction) before analysis, for example in SNAP. |

## 3.4 Nigeria Airports (HDX)

| Source | Humanitarian Data Exchange (HDX); original publisher listing: World Bank energydata.info – Nigeria Airports |
| --- | --- |
| Format / licence | Shapefile (zipped). The published licence is Creative Commons Attribution 4.0 (CC BY 4.0). |
| Number of features | [N] point features (read from the layer's feature count in QGIS: right-click the layer, then Show Feature Count). |
| Geometry type | Point |
| Key columns | According to the publisher, the dataset holds the airport name and no other attributes. Fields in QGIS: [list field names, e.g. name] |
| CRS | [e.g. EPSG:4326 – confirm in Layer Properties] |
| Gaps and issues | There are no attributes beyond the name, so there is no airport type, runway, status or operator. The file dates from about 2017, so new or closed airfields may be missing or out of date. Missing/NULL names: [count, if any]. |

## 3.5 Nigeria Administrative Boundaries (GRID3)

| Source | GRID3 Data Hub; programme site: grid3.org |
| --- | --- |
| Dataset / version | [exact layer name, e.g. Nigeria – Administrative Boundaries (state/LGA/ward) and download date] |
| Number of features | [N] polygons. For reference, Nigeria has 36 states plus the FCT and 774 LGAs; check which level you downloaded: [state / LGA / ward] |
| Geometry type | Polygon / MultiPolygon |
| Key columns | [list from the attribute table, e.g. state name, LGA name, state code, LGA code] |
| CRS | [e.g. EPSG:4326] |
| Gaps and issues | Check for NULL or empty names/codes: [count]. Check for invalid or overlapping geometries (Vector > Geometry Tools > Check Validity): [result]. Boundaries in the delta and coastal areas can differ from other sources. |

## 3.6 Nigeria Administrative Boundaries (DIVA-GIS)

| Source | DIVA-GIS country data (Country: Nigeria, Subject: Administrative areas) |
| --- | --- |
| Format / levels | Shapefile with admin levels 0 (country), 1 (state), 2 (LGA). Levels used: [levels] |
| Number of features | [N] polygons per level used (e.g. level 1: [N]; level 2: [N]) |
| Geometry type | Polygon / MultiPolygon |
| Key columns | Typical DIVA-GIS (GADM-derived) fields include ID_0, ISO, NAME_0, ID_1, NAME_1, ID_2, NAME_2, TYPE_2 and ENGTYPE_2. Confirm against the actual attribute table: [actual fields] |
| CRS | Geographic WGS84 (EPSG:4326) – confirm in Layer Properties |
| Gaps and issues | This is a generalised dataset and is older than GRID3, so LGA boundaries may be out of date or less precise. Compare the feature counts with GRID3 to see differences: [observations] |

# 4. Opening the Data in QGIS

- Rasters (Landsat, Sentinel-2, Sentinel-1) were added with Layer > Add Layer > Add Raster Layer and checked for band count, CRS and extent.

- Vectors (airports, GRID3, DIVA-GIS) were added with Layer > Add Layer > Add Vector Layer. Attribute tables were opened to confirm the fields and feature counts.

- All layers were checked against one project CRS ([e.g. EPSG:32632 / EPSG:4326]) and loaded to confirm they overlap the study area.

- The QGIS project file (.qgz) and a screenshot of the loaded layers are included in the repository.

Screenshot: [insert QGIS screenshot here]

# 5. Overall Observations on Gaps

- Optical imagery (Landsat, Sentinel-2) is affected by cloud cover, so Sentinel-1 radar is used as a complement.

- The airports layer has names only, with no descriptive attributes, and may be dated.

- The two admin boundary sources (GRID3 and DIVA-GIS) may disagree on feature counts and boundaries, so one will be chosen as the primary layer and the other kept for cross-checking.

- Large raster files are not stored directly in GitHub; only metadata, download links and small samples are committed (see Section 6).

# 6. Repository Contents

Suggested structure for the GitHub repository:

- README.md – project summary and link to this data note

- docs/data_note.docx (or data_note.md) – this document

- data/raw/ – small vector files (airports, admin boundaries) with source links

- data/metadata/ – Landsat MTL files and Sentinel scene lists (IDs, dates, cloud cover)

- qgis/ – the .qgz project file

- images/ – QGIS screenshots

- .gitignore – excludes large rasters (.tif, .SAFE, .zip)

# 7. Source Links

- USGS EarthExplorer (Landsat 8/9): https://earthexplorer.usgs.gov/

- USGS Landsat Missions: https://www.usgs.gov/landsat-missions

- Copernicus Browser (Sentinel-1 and Sentinel-2): https://browser.dataspace.copernicus.eu/

- ESA Sentinel-1 mission: https://sentinels.copernicus.eu/copernicus/sentinel-1

- ESA Sentinel-2 mission: https://sentinels.copernicus.eu/copernicus/sentinel-2

- HDX – Nigeria airports search: https://data.humdata.org/dataset?q=nigeria+airports

- World Bank energydata.info – Nigeria Airports: https://energydata.info/dataset/nigeria-airports

- GRID3 Data Hub: https://data.grid3.org/

- DIVA-GIS country data: https://www.diva-gis.org/gdata

- Project GitHub repository: [paste URL]