GeoDev – Week 3

QGIS Data Preparation: Coordinate Systems Done Right

Class summary and study notes  |  Seigha Tukoroa  |  18 October 2026

# 1. The Big Picture

A map can look perfect on screen while the numbers behind it (areas, distances, overlays) are wrong. This class is about preparing data properly before any analysis: knowing what coordinate system each layer uses, converting it correctly, checking quality, clipping cleanly and saving in a good format.

## The seven topics at a glance

| Topic | One-line takeaway |
| --- | --- |
| Inspecting a layer's CRS | Know the coordinate system of every layer before you use it. |
| Reading EPSG codes | An EPSG code is a short ID for a coordinate system; learn the few you will use most. |
| Reproject vs assign | Reproject changes the coordinates. Assign only changes the label. |
| Area in square metres | Reproject to a metre-based projected CRS first, then calculate. |
| Raster Calculator | Do maths on raster bands (for example NDVI) to make new rasters. |
| Data quality questions and clipping | Check the data before trusting it; clip only after CRS and geometry are clean. |
| GeoPackage vs Shapefile | Save work as GeoPackage (.gpkg), a modern single-file format. |

# 2. Inspecting a Layer's Coordinate Information

- Double-click the layer, then open Properties > Information (shows CRS, extent, geometry type, feature count) and Source (assign or change the CRS).

- The CRS button at the bottom-right of the QGIS window shows the project CRS, which controls display only.

- A layer's CRS and the project CRS can be different. QGIS reprojects layers on the fly so they line up on screen, but this does not change the stored data.

# 3. Reading EPSG Codes

An EPSG code is a numeric ID for a coordinate reference system (CRS), written in QGIS as EPSG:4326. A CRS has two parts: a geographic part (datum and ellipsoid) and, if projected, a map projection that converts the curved earth to a flat plane with units in metres.

| EPSG | Name | Units | Use |
| --- | --- | --- | --- |
| 4326 | WGS 84 (geographic) | Degrees | Default for GPS, downloads and data exchange. Not for measuring. |
| 3857 | Web Mercator | Metres | Web basemaps only. Distorts area badly away from the equator. |
| 32631 | WGS 84 / UTM zone 31N | Metres | Good for measuring in the western Niger Delta (about 0° to 6°E). |
| 32632 | WGS 84 / UTM zone 32N | Metres | Good for measuring over eastern Nigeria (about 6° to 12°E). |
| 26391 to 26393 | Minna / Nigeria West, Mid and East Belt | Metres | Older Nigerian survey data; many local datasets use these. |

- Geographic CRS: coordinates in degrees (latitude and longitude). One degree is not a fixed distance.

- Projected CRS: coordinates in metres on a flat grid, which is what you need for area and distance.

- Pick a UTM zone or national CRS that covers your study area. Nigeria sits across UTM zones 31N and 32N, so check which zone your data falls in.

VITAL POINT: EPSG:4326 is for storing and sharing. A projected metre-based CRS is for measuring.

# 4. Reprojecting vs Assigning a Coordinate System

This is the most common beginner mistake, and the class stressed it.

|  | Reproject | Assign (define) projection |
| --- | --- | --- |
| What it does | Transforms the coordinate values to a new CRS. | Only sets the CRS label; coordinates are unchanged. |
| When to use | The layer already has the correct CRS and you want a different one. | The CRS is missing or wrong (for example a missing .prj file). |
| QGIS tool | Vector > Data Management Tools > Reproject Layer (raster: Raster > Projections > Warp (Reproject)) | Vector > Data Management Tools > Assign Projection (or Layer Properties > Source > Assigned CRS) |
| Result | New layer with new coordinate numbers. | Same coordinates, new label. |
| Risk | Low if the original CRS is correct. | Wrong assignment puts data in the wrong place, sometimes in the ocean or at 0,0. |

VITAL POINT: Assign when the CRS is unknown or mislabelled. Reproject when the CRS is known and correct and you want to convert. Never assign a CRS to "fix" a layer that already has the right one.

Quick test: if coordinates look like 5.7, 6.2 they are probably degrees (EPSG:4326). If they look like 500000, 650000 they are probably metres in a projected CRS.

# 5. Calculating Area in Square Metres

- Check the layer CRS. If it is EPSG:4326, areas will come out in square degrees, which are meaningless.

- Reproject the layer to a projected CRS in metres (for example EPSG:32631 or 32632).

- Open the Field Calculator (Ctrl+I). Create a new decimal field, for example area_m2.

- Use the expression area($geometry) or $area. $area uses the project's ellipsoid settings; area($geometry) is planar in the layer's own units.

- Convert if needed: divide by 1,000,000 for km², or by 10,000 for hectares.

VITAL POINT: Reproject first, calculate second. The same polygon gives completely different areas depending on the CRS used.

# 6. Using the Raster Calculator

Open it from Raster > Raster Calculator. It does band maths and writes the result as a new raster.

- Band names look like "B5@1" (layer name, then band number).

- Set the output layer, output format, extent and CRS carefully, and check the expression result is valid before running.

- Example, NDVI for Landsat 8/9: ("B5@1" - "B4@1") / ("B5@1" + "B4@1") (B5 = near-infrared, B4 = red).

- Example, NDVI for Sentinel-2: ("B8@1" - "B4@1") / ("B8@1" + "B4@1") (B8 = near-infrared, B4 = red).

- Conditions give masks (1/0), for example "NDVI@1" > 0.3 to flag dense vegetation.

# 7. The Five Data Quality Questions

Before analysing any dataset, ask these five questions. (The wording below is a standard set; check it against your class slides and adjust if your lecturer's list differs.)

| # | Question | What to check in QGIS |
| --- | --- | --- |
| 1 | Where did it come from? | Source, publisher and licence. Record the link in your data note. |
| 2 | How current is it? | Acquisition or publication date. Is it recent enough for your question? |
| 3 | What CRS is it in, and is that right? | Properties > Information. Does it overlap other layers where expected? |
| 4 | Is it complete? | Feature count, NULL or empty attributes, gaps, cloud cover, missing areas. |
| 5 | Is it accurate enough for my purpose? | Scale and resolution, valid geometries (Vector > Geometry Tools > Check Validity), and whether the detail matches your question. |

# 8. Clipping Cleanly

- Make sure the layers to clip and the clip layer use the same CRS (reproject first if not).

- Fix invalid geometries first (Vector > Geometry Tools > Check Validity, then Fix Geometries).

- Vector: Vector > Geoprocessing Tools > Clip (input layer, overlay layer = boundary).

- Raster: Raster > Extraction > Clip Raster by Mask Layer. Set a NoData value and decide whether to match the mask extent.

- Check the result: feature count, extent, and that the edges follow the boundary. Save the output with a clear name.

VITAL POINT: A clip is only as good as its inputs: matching CRS and valid geometry first, then clip. Always inspect the output.

# 9. Save as GeoPackage, Not Shapefile

|  | GeoPackage (.gpkg) | Shapefile (.shp) |
| --- | --- | --- |
| Files | One single file | Several files (.shp, .shx, .dbf, .prj and more); easy to lose one |
| Field names | Long names allowed | Limited to 10 characters |
| Size limit | Very large (far above 2 GB) | About 2 GB per file |
| Content | Multiple vector layers (and rasters) in one file | One layer, one geometry type |
| Text and NULLs | Full Unicode; proper NULL values and date/time types | Limited; weaker NULL handling |
| Overall | Modern, open standard recommended by QGIS | Old format, still common but limiting |

How: right-click the layer > Export > Save Features As... > Format: GeoPackage, set the file name, layer name and CRS.

# 10. Data Prep Workflow Checklist

- Add the data and inspect each layer's CRS and metadata.

- Fix any missing or wrong CRS with Assign Projection (only when it is genuinely unknown or wrong).

- Choose one projected, metre-based CRS for the study area (for example EPSG:32631 or 32632).

- Reproject all layers to that CRS (Reproject Layer for vectors, Warp for rasters).

- Answer the five data quality questions for every dataset.

- Check and fix invalid geometries.

- Clip to the study area.

- Calculate areas (Field Calculator) and rasters (Raster Calculator) as needed.

- Save results as GeoPackage and commit them to GitHub with a short note.

# 11. Common Mistakes to Avoid

- Calculating area or distance in EPSG:4326 (degrees).

- Using Assign Projection when you needed Reproject, or the other way round.

- Trusting the project CRS instead of checking each layer's own CRS.

- Using Web Mercator (EPSG:3857) for measurement.

- Clipping before matching CRS and checking for invalid geometry.

- Saving to shapefile and losing long field names or a sidecar file.

# 12. Applying It to Your Project Data

- Landsat 8/9, Sentinel-1 and Sentinel-2 rasters: check each file's UTM zone, reproject to one common CRS and use the Raster Calculator for NDVI.

- GRID3 and DIVA-GIS admin boundaries and the HDX airports: confirm they are in the same CRS, reproject to your chosen UTM zone and compute areas in m² or km².

- Clip all layers to your study area (for example the Niger Delta corridor) and save the results in a GeoPackage in the repository's data folder.