GeoDev – Month 01 Integration Task

Four Weekly Tasks, One Repository, One Story

Seigha Tukoroa  |  19 October 2026

GitHub repository: [https://github.com/seighatukoroa/geodev-lab-project.git]

# 1. Month Summary at a Glance

| Week | Theme | What you produced |
| --- | --- | --- |
| 1 | Project brief | Project brief with a source link for every dataset. [Which wards in Burutu LGA, Delta State are more than 5km from a health facility?] |
| 2 | Data download and notes | Landsat 8/9 (USGS), Sentinel-1 and 2 (Copernicus), HDX airports, GRID3 and DIVA-GIS admin boundaries, opened in QGIS and described in a data note. |
| 3 | Data preparation | CRS checks, reprojection, area in m², Raster Calculator, five data quality questions, clipping, GeoPackage output. |
| 4 | Spatial analysis | 200 m waterway buffer and intersection with settlements in Ogbia wards, four-way result check, map PNG and month-1-summary.md. |

# 2. The Four Weekly Records

## Week 1: Project Brief

| Item | Your entry |
| --- | --- |
| Place | Burutu LGA, Delta State. |
| Your question (one sentence) | Which wards in Burutu LGA, Delta State are more than 5km from a health facility? |
| Why it matters | This identifies wards in Burutu LGA, particulary in the largest mass area of Obotebe ward, that displayed Distance is one of the biggest barriers to care, especially for pregnant women and children. Mapping how many people live far from a clinic shows exactly where new facilities or mobile clinics will save the most lives. This hightlight gaps, and unequal physical areas. |
| Datasets planned | The data I need - Nigeria Ward Level boundaries – GRID 3 – https://data.grid3.org/datasets/GRID3::grid3-nga-operational-wards-v3-0/about – 128 mb - Nigeria LGA boundaries – GRID 3 – https://data.grid3.org/datasets/GRID3::grid3-nga-operational-lga-boundaries/about – 2.6 mb - Nigeria State boundaries – GRID 3 – https://data.grid3.org/datasets/GRID3::grid3-nga-operational-state-boundaries-/about – 645 kb - Health Facilities in Burutu LGA, Delta State: GRID3 NGA – Health Facilities v3.0 (September 2026) – 4.7 mb. - The GRID3 NGA, Health Facilities v3.0 dataset is a non-exhaustive, non-validated geographic representation of health facility points. https://data.grid3.org/datasets/827e3638dc204f4b9ddbbd19b00954d6/about - Road Data for Burutu LGA, Delta State: Extracted using QuickOSM plugin from QGIS software. |
| Method planned | [What you would build. An interactive web map of burutu, that shows every ward coloured by it distance to the nearest health facility, with those beyond 5 km clearly flagged in ed. The map would and can be updated each month as new facility or road data is added. Local health official can open this map on any phone or computer. EPSG:32632 – WGS 84 / UTM zone 32N] |
| File in repository | week1-project-brief/project_brief.md [or .docx/.pdf] |

## Week 2: Data Note

Datasets downloaded and opened in QGIS, each described with source, feature count, key columns, geometry type and gaps.

| Dataset | Source | Link |
| --- | --- | --- |
| Landsat 8/9 | USGS EarthExplorer | earthexplorer.usgs.gov |
| Sentinel-2 and Sentinel-1 | Copernicus Data Space | browser.dataspace.copernicus.eu |
| Nigeria airports | HDX / World Bank | energydata.info Nigeria Airports and HDX search |
| Admin boundaries | GRID3 | data.grid3.org |
| Admin boundaries | DIVA-GIS | diva-gis.org/gdata |

File in repository: week2-data-note/data_note.docx (the document I drafted earlier).

Key gaps noted: cloud cover on optical imagery; airports layer has names only and is dated; GRID3 and DIVA-GIS boundaries may disagree. Add the actual feature counts from QGIS: [counts]

## Week 3: Prepared Data and Quality Checks

Data preparation rules applied, with the checks you ran. Tick or fill each for your own data.

| Preparation step | Your record |
| --- | --- |
| Inspected each layer's CRS | [WGS 84 / UTM zone 32N] |
| Chosen projected CRS for analysis | [EPSG:32632 – WGS 84 / UTM zone 32N] |
| Reprojected (not just assigned) | Reproject-Assign (define) projection What it does- Transforms the coordinate values to a new CRS. Only sets the CRS label; coordinates are unchanged. When to use- The layer already has the correct CRS and you want a different one. The CRS is missing or wrong (for example a missing .prj file). QGIS tool- Vector > Data Management Tools > Reproject Layer (raster: Raster > Projections > Warp (Reproject)) Vector > Data Management Tools > Assign Projection (or Layer Properties > Source > Assigned CRS) Result New layer with new coordinate numbers. Same coordinates, new label. |
| Area calculated in square metres | - Check the layer CRS. If it is EPSG:4326, areas will come out in square degrees, which are meaningless. - Reproject the layer to a projected CRS in metres (EPSG: 32632). - Open the Field Calculator (Ctrl+I). Create a new decimal field, for example area_m2. - Use the expression area($geometry) or $area. $area uses the project's ellipsoid settings; area($geometry) is planar in the layer's own units. Convert if needed: divide by 1,000,000 for km², or by 10,000 for hectares |
| Raster Calculator used | Band names look like "B5@1" (layer name, then band number). - Set the output layer, output format, extent and CRS carefully, and check the expression result is valid before running. - Example, NDVI for Landsat 8/9: ("B5@1" - "B4@1") / ("B5@1" + "B4@1") (B5 = near-infrared, B4 = red). - Example, NDVI for Sentinel-2: ("B8@1" - "B4@1") / ("B8@1" + "B4@1") (B8 = near-infrared, B4 = red). - Conditions give masks (1/0), for example "NDVI@1" > 0.3 to flag dense vegetation. |
| Clipped to study area | - Make sure the layers to clip and the clip layer use the same CRS (reproject first if not). - Fix invalid geometries first (Vector > Geometry Tools > Check Validity, then Fix Geometries). - Vector: Vector > Geoprocessing Tools > Clip (input layer, overlay layer = boundary). - Raster: Raster > Extraction > Clip Raster by Mask Layer. Set a NoData value and decide whether to match the mask extent. - Check the result: feature count, extent, and that the edges follow the boundary. Save the output with a clear name. |
| Saved as GeoPackage | Files-One single file Several files (.shp, .shx, .dbf, .prj and more); easy to lose one Field names Long names allowed Limited to 10 characters Size limit Very large (far above 2 GB) About 2 GB per file Content-Multiple vector layers (and rasters) in one file One layer, one geometry type Text and NULLs Full Unicode; proper NULL values and date/time types Limited; weaker NULL handling Overall Modern, open standard recommended by QGIS Old format, still common but limiting |

Five data quality questions: source, currency (date), CRS, completeness, and accuracy/fitness for purpose. (This is the standard set I used; check it against your class slides.) Record your answer for each dataset in week3-data-prep/quality_checks.md.

| Dataset | Source | CRS | Complete? | Fit? |
| --- | --- | --- | --- | --- |
| Ward_Boundaries | [data.grid3.org] | [EPSG:32632 – WGS 84 / UTM zone 32N ] | [ YES] | [Government ministry ] |
| Waterway_ogbia | [ DivaGIS] | [EPSG:32632 – WGS 84 / UTM zone 32N ] | [ YES] | [Ministry of environment] |
| Settlement_Extents_utm | [data.grid3.org ] | [EPSG:32632 – WGS 84 / UTM zone 32N ] | [ YES] | [Ministry of environment] |

## Week 4: Analysis, Map and month-1-summary.md

| Item | Record |
| --- | --- |
| Operation | 200 m buffer on Waterway_ogbia, intersected with Settlement_Extents (settlements classed within / outside 200 m of water). |
| Projected CRS | [EPSG:32632 – WGS 84 / UTM zone 32N] |
| Expected before running | [Ogbia is a low-lying riverine area with many creeks, so many settlements sit by water]] |
| Four-way check | [map view / row count / one feature verified by hand / empty geometry: results] |
| Map image | week4-analysis/ogbia_settlements_200m_waterways.png (title, legend, scale bar) |
| Summary file | month-1-summary.md (see the Week 4 document, Section 9) |

# 4. README.md Draft

The README must link to each week so a mentor does not have to hunt through folders. Copy this into README.md and replace the yellow items.

# Ogbia Settlements and Waterways: Month 1 Project

**Question:** [Which settlement in ogbia LGA sit in low lying land within 200m of a watercourse]

**Answer in one line:** Which settlement in ogbia LGA sit in low lying land within 200m of a watercourse. Intersection 824 out of 1078 settlement.]

**Place:** [Ogbia LGA, Bayelsa State, Nigeria]  |  **Tools:** QGIS, GitHub  |  **CRS:** [EPSG code]

## Contents

- Week 1: [Project brief](week1-project-brief/project_brief.md), with a source link for every dataset

- Week 2: [Data note](week2-data-note/data_note.docx), describing what was downloaded

- Week 3: [Prepared data](week3-data-prep/) and [quality checks](week3-data-prep/quality_checks.md)

- Week 4: [Analysis](week4-analysis/), [map](week4-analysis/ogbia_settlements_200m_waterways.png) and [month-1-summary.md](month-1-summary.md)

## How the weeks connect

Week 1 framed the question and datasets; Week 2 downloaded and described the data; Week 3 put it in a projected CRS and checked its quality; Week 4 ran the analysis that answers the question.

## Result

![Map](week4-analysis/ogbia_settlements_200m_waterways.png)

## Limits and next steps

[data gaps, what you still need, e.g. elevation/flood extent, newer waterway lines]

# 5. How the Four Weeks Connect as One Project

The marker gives 3 marks for the weeks reading as a single story. Say it explicitly in the README and in the summary below:

- Week 1 sets the place and question (settlement exposure to waterways in Ogbia).

- Week 2 collects the data to answer it: boundaries for wards, imagery for context, reference layers.

- Week 3 makes the data comparable: one projected CRS, clean geometry, clipped to the study area, saved as GeoPackage.

- Week 4 uses the prepared data to run the buffer and intersection, checks the result and maps it.

- Use the same place name, layer names and CRS everywhere so a reader can follow one thread.

# 6. Your Own Question, Answered

| Item | Your entry |
| --- | --- |
| Question | [e.g. Which settlements in Ogbia lie within 200 m of a waterway?] |
| Answer (one or two plain sentences) | [e.g. X of Y settlement areas (Z%) lie within 200 m of a waterway. Wards A and B have the highest share.] |
| Evidence | [counts, area in km², the map, the four-way check results] |
| Confidence and limits | [e.g. waterway layer incomplete for small creeks, settlement extents dated, 200 m is a chosen distance, not a measured flood limit] |
| Still needed | [e.g. DEM/flood data, population per settlement, newer imagery] |

# 7. Repository Structure and Commit History

Suggested layout:

ogbia-project/

README.md

month-1-summary.md

week1-project-brief/project_brief.md

week2-data-note/data_note.docx

week3-data-prep/quality_checks.md

week3-data-prep/ogbia_prepared.gpkg

week4-analysis/ogbia_analysis.gpkg

week4-analysis/ogbia_settlements_200m_waterways.png

qgis/ogbia_project.qgz

.gitignore   (exclude large rasters such as .tif, .SAFE, .zip)

Commit history (2 marks): the history should show the work week by week, not one giant upload. If you already committed everything at once, make the next commits meaningful and separate (README, fixes, results). Do not rewrite or fake dates. Example messages:

git commit -m "Add week 1 project brief"

git commit -m "Add week 2 data note and source links"

git commit -m "Add week 3 prepared data and quality checks"

git commit -m "Add week 4 analysis, map and summary"

git commit -m "Add month 1 summary"

git commit -m "Add README linking all four weeks"

NOTE: Check reachability: open the repository link in a private window and click every README link. Broken relative links lose the 5 and 4 mark criteria. Large files over 100 MB cannot go to GitHub, so keep rasters out and describe them in the data note.

# 8. Submission Checklist

- Week 1 brief present, with a source link for every dataset (5 marks across all four weeks)

- Week 2 data note present

- Week 3 prepared data and quality checks present

- Week 4 analysis, map image and month-1-summary.md present

- README links to each week, states the question and plain answer (4 marks)

- Own question answered with a real result about your chosen place (6 marks)

- The four weeks tell one story with consistent names and CRS (3 marks)

- Commit history shows the work in stages (2 marks)

- On the assignment: Add or create > Link > paste the repository URL > Turn in (not the private comment box)

- The task shows as Missing with a due date of 30 September, so submit as soon as possible and check with your instructor about late work.